# Tides in basins and estuaries

# Name: Peter, Mattia & Sam

Studentnumber: Insert studentnumber (solis-ID) here

The questions in this exercise are formulated to guide your analysis of tides in estuaries and basins. In the answers to your questions make sure you show that you understand the main issues. Do not focus on generating as many plots as possible.

Part 1: Dynamics of rectangular tidal basins

We start with a rectangular basin (having constant width) of 1 km wide.

Discuss and explain your results:

- Take a basin of 20 km length. Take Cd=2.5 ∙ 10-3. Take deltaX=500 m. Prescribe an M2 tide at the seaward side with an amplitude of 1 meter. Vary the still water depth between 2 and 10 meters in steps of 1 meter. Make sure the time step is small enough by checking the Courant number for the case with largest depth (CFL<1). This will also result in a good time step for the shallower cases.
1)	Study the sensitivity of the amplitude of the M2 tide as function of

space for the different depths. Explain the dependence of tidal amplitude on depth. For which depths are you close to the pumping model solution (Short basin limit in paper by Friedrichs)? Also remember that for pumping model the phase of M2 water level should be uniform in the basin.

Insert your answer to (1) here. Use a maximum of 2 figures and keep your text brief. In order to make the Courant number small enough we had to set DeltaT at 45. M2 tidal amplitude increases landward for heights above 3m. This is due to the negative correlation between friction and tidal amplitude. The shallower the water, the higher the levels of friction and so the more dampened is M2.

2) 	Determine the deformation of the tide for the shallowest case (2 m) and deepest case (10 m) by determining the amplitude of the M4 water level. Study these as function of position in the basin. Explain the difference between a deep and shallow basin.

Insert your answer to (2) here. Use a maximum of 1 figure and keep your text brief. The M4 amplitude is maximal in a shallow basin (H = 2/3 meters) and the amplitude decreases when the depth of the basin increases till a basin depth of 9 meters. At a basin depth of 10 meters the effect of the M4 is completely diminished. Is this still due to friction?!

3) 	Do this one after the third lecture (Nov 30). For the shallowest case: Determine the amplitude of the mean (=averaged over a tidal cycle), the M2 and the M4 tidal currents in the basin and determine the relative phase difference between the M2 and M4 tidal currents. Explain (i) the presence of the mean tidal currents and (ii) explain why peak ebb or peak flood currents are largest (for different positions in the basin).

Insert your answer to (3) here. Use a maximum of 3 figures and keep your text brief. We only have this for M2, right?!

- Take a basin with depth of 10 meters, deltaX=1000 m. Choose an appropriate time step. Prescribe an M2 tide at the seaward side with amplitude of 1 meter.

- Vary the length of the basin between 10 km and 150 km in steps of 20 km and take Cd=0.2 ∙ 10-3. Study the sensitivity of the amplitude of the M2 tide as function of space for the different lengths of the basin. Explain your results. In which case is the basin resonant? Compare to what you expect theoretically.

Insert your answer to (1) here. Use a maximum of 1 figure and keep your text brief. We observe from the figure above that the amplitude of the M2 water level grows inland for every basin length and depth H0=10. This is consistent with what we argued in Part A. Furthermore, amplitude across the basin is maximised for sizes of around 110 meters. Theoretically, we expect resonance when the "quarter wave length" criteria is required, meaning that L=lambda/4. When the basin is that long, amplitude and speed grow infinitely if friction is ignored as the standing waves will "pile up" on top of each other over time. Because the pumping approximation is met (see Lk), meaning that the amplitude phase of the wave is approximately constant across the basin, we know that the wavelength of M2 will also be constant for different lengths of the basin. Therefore, we can find the L such that L=lambda/4 graphically. We note that this must be somewhere around 110km, and more specifically it will be around 104km long.

- Take a basin length of 110 km and study the M2 water level amplitude in the basin for Cd=[0.2 ∙ 10-3, 0.5 ∙ 10-3, 10-3, 2 ∙ 10-3, 5 ∙ 10-3]. Explain your results. What happens to the wave length of the tide due to friction? In case of moderate friction, is the M2 tide resonant for larger basin lengths or smaller basin lengths compared to the case of negligible friction? Give two reasons why the amplitude of the M2 water levels at the end of the basin is smaller for increasing values of Cd.

Insert your answer to (2) here. Use a maximum of 1 figure and keep your text brief. Let's assume that moderate friction and negligible friction are represented by the cases where c_d = 0.0010 and c_d = 0.0002 respectively. For moderate friction, it appears as though the M2 tide is resonant for shorter basin lengths. Specifically, in the case of negligible friction, resonance is reached after approx. 104 km. Then, in the case of moderate friction, resonance is reached after approx. 90 km. This is due to the fact that friction shortens the wavelength. The amplitude of the M2 waterlevels is smaller at the end of the basin for increasing values of Cd because of: 1. Higher friction damps the signal (energy loss). Both decreased amplitude and speed 2. In this specific case, friction impacts resonance too, as now resonance is only present in a shorter basin.

- Closure of Zuiderzee. Take a basin depth of 5m, Cd=2 ∙ 10-3, deltaX=1000m. Study M2 sea surface height amplitude and phase for the case that the basin is 110 km long (before closure) and for the case that the basin is 40 km long. Explain why for the shorter tidal basin the Tidal Prism is larger than the case that the basin was long. Take two effects into account: The tidal wave propagation and resonance effects. The tidal prism (TP) is calculated as:

TP=0.50T|Qx=0,t|dt

Insert your answer to (C) here. Use a maximum of 2 figures and keep your text brief. After computing the following TP values were returned: TP1 (40k) = 1.29e9 & TP2 (110k) = 9.32e8. This value (tidal prisms) tells something about the volume of water in an estuary or an inlet between mean high tide and mean low tide. With high surface drag coefficient, M2 is "killed" early, and so is the effect of resonance. As the length of the basin is bigger when the estuary is not closed, there will be less tidal prism in the latter case. With friction, movement is slow and long basin has more length to cover. Another point to note is from figure B2_b we noted that the amplitude and speed of M2 are not majorly dampened in the shorter basin. There is a likelihood that friction plays a bigger role over bigger lengths. The tidal prism is lower in the 110km basin because when observing the graph we note that quarter wave length is obtained at 40km. In the shorter basin, phase doesn't vary meaning it's a standing wave whereas in the longer basin, the phase varies meaning that you will have a lower and higher tide across the length. This explains the fact that the tidal prism is bigger in the shorter basin. More water is displaced overall in the shorter basin (also noticeable with the wave amplitude in the figure).

Part 2: Dynamics of estuaries

For this case we take a converging channel width, as often observed in real estuaries. The width is modeled as:

Where B0 is the width at the entrance of the estuary and Lb is the e-folding length scale of the width. At some point the channel gets a constant width Briver determined by the dynamics of the river.

Discuss and explain your results:

- Solve the tidal dynamics in your own estuary as explained in Matlab2.pdf

  - Vary the bed level and try to find the depth for which the estuary is an equilibrium estuary. Show and discuss your results for three depths: equilibrium depth, a much shallower system (take 66% of the equilibrium depth) and much deeper system (take 150% of equilibrium depth).

    - Explain the sensitivity of M2 water level amplitude to depth.

    - Explain the increase in tidal amplitude at the landward end of the estuary.

    - Determine and explain the phase speed (=propagation speed of the wave; note that phase behaves as -kx, with k the wave number and that c=angular velocity/wave number) of D2 tide (and D1 tide if you have mixed tide) and its sensitivity to water depth. Also discuss the deviations from c=sqrt(gh).

Insert your answer to (1) here. Use a maximum of 2 figures and keep your text brief. For the case of the Gironde Estuary: - B_0 = width at the entrance of the estuary (defined as a relatively narrow constriction point before reaching the open sea) = 5.8km. - Briver = width of the river where the estuary stops narrowing. Unfortunately, it is important to note that in our case it seems like the estuary splits in two before this happens. The Dordogne and Garonne rivers seem to stop narrowing at 107km and 111km so that at110km the width is = 0.25km. The yearly averaged semi-diurnal tides are approximately 15x the value of the diurnal tides. Therefore, we use only semi-diurnal tides in this analysis. Equilibrium depth is found at H0=8.5m. With this depth the estuary is at equilibrium (the phase of the M2 amplitude is constant) up to 82.5km inland, hence, around 85% of point in river at which estuary stops narrowing. D1i. Shallow waters: equilibrium is not reached as friction is dominant over convergence and dampens the amplitudes across the basin. Equilibrium figure: the two factors are in balance such that the amplitude is constant up to 75% of the length of xr. Deep waters: equilibrium is not reached either because the tidal convergence dominates over friction. D1ii. After xr, the width is constant, and river dynamics play a dominant role. Hence, we expect higher water levels due to water displacement. [!!!!] This is due to resonance (look at M4, M6) - this is explained in previous parts as you look at length x-xr. D1iii. C = 7.2680 9.9128 14.2718 C_th = 7.4185 9.1315 11.1838 The propagation speed according to -kx is similar to that which was calculated theoretically from the sqrt(gh). It seems like it is slower than the theoretical value, possibly due to friction effects. On the other hand, it is faster than the the theoretical value for the equilbrium value and deep value. In the former case, it is just slightly faster (approx 10%), while for the latter, it is significantly faster (approx 30%). This seems to suggest that the channel convergence has a quickening effect on the velocity. Compare to paper's function for c and explain the crossing by looking at Lb and La.

  - For the case with equilibrium depth, compare the modeled wave number (k), propagation speed of wave (C), velocity magnitude (U), and phase difference (φ) between water levels and flow velocities with the theoretically predicted values presented in section 3.10 in Friedrichs (2010).

Insert your answer to (2) here. Use a maximum of 3 figures and keep your text brief. k = 1.6748e-05 k_t = 1.5393e-05 c = 8.3928 c_t = 9.1315

- For the case of equilibrium depth: determine the magnitude of the mean flow along the estuary.  Explain your results.

Insert your answer to (3) here. Use a maximum of 2 figures and keep your text brief. Average speed starts from zero at sea boundary and increases up to around 90% of our estuary, and then slows back down to zero as we enter the river of constant width. This means that across time, tidal flow is fastest at around 95km inland. This is not surprising??? Also understand why phaseUM2 behaves like that jumping. This value is the ratio of discharge to cross sectional area. This figure suggests that the ratio increases up to almost the end of the e-folding length scale. This means that there is higher water flow per volume at the landward portion of the estuary which makes sense as we have higher amplitudes? Or as we are at high tide??

- Take values of your optimized case. Study the sensitivity of (i) M2 water level amplitude, (ii) M2 velocity amplitude and (iii) propagation speed of tidal wave to e-folding length scale of channel width (Take range between Lb 50% larger and Lb 33% smaller). When changing Lb, also change B0 by using that B0=Briver*exp(xr/Lb).  Explain your results.

Insert your answer to (4) here. Use a maximum of 3 figures and keep your text brief.